RLLS 16

Master Product and Technical Design Document

Version 0.3  |  3D Artificial-World Simulation System
Implements: Chunk Streaming, Frustum Culling, Deep Zoom, GPU Buffer Pooling
Status: Implementation Complete — All Tests Passing

# Table of Contents

This document is organized into six parts:

Part I    — Product Requirements Document (PRD)

Part II   — Technical Architecture Document

Part III  — Security & Data Integrity Specification

Part IV   — Frontend Specification

Part V    — Chunk Streaming & Deep Zoom Architecture

Part VI   — Feature Ticket List & Engineering Rules

# PART I — PRODUCT REQUIREMENTS DOCUMENT

## 1. Product Definition

RLLS16 is a desktop digital-Earth simulation workstation in which the user can move continuously from Solar-System scale to near-ground inspection of a single persistent canonical Earth. The system decouples an immutable authoritative planetary dataset from mutable simulation instances and reinforcement-learning episodes. Rendering is hardware-accelerated via OpenGL 3.3 Core / ModernGL, operating on Windows with multi-GPU support (NVIDIA Optimus / AMD PowerXpress).

## 2. Product Vision

Create one persistent artificial Earth that can be observed, navigated, simulated, queried, and used as a reinforcement-learning environment — all from a single cohesive application. The Earth's terrain, oceans, atmosphere, and ecosystem respond to time, solar input, and user actions without ever re-generating the canonical planetary dataset.

## 3. Product Principles

One authoritative canonical Earth; do not regenerate the planet for each simulation world.

Aggregate globally, individualize locally: cohort fields at large scale; individual objects only when spatially relevant.

Deep zoom is a data/LOD problem, not a camera-distance problem.

Only the visible/nearby spatial region should consume high-end rendering resources (chunk streaming).

GPU rendering is optional for the simulation/data layer but required for normal interactive 3D display.

Determinism is a first-class requirement for terrain, vegetation, simulation and RL reproducibility.

Infrastructure uses mature libraries when demonstrably safer or more efficient than hand-written equivalents.

No feature is accepted because code compiles; runtime telemetry and automated tests must validate it.

## 4. Target Users

Simulation researchers: observe coupled Earth-system dynamics over geological time scales.

RL practitioners: train policies in a physics-coupled environment with reproducible canonical state.

Game/visualization developers: explore spatial rendering strategies at planetary-to-ground scale.

Educators: demonstrate Earth-science concepts interactively in a real-time 3D environment.

## 5. Core User Journeys

Create World: choose name, population, environment archetype, RL goal; generate world instance without regenerating canonical Earth.

Explore Earth: focus Earth, zoom continuously, rotate/pan, descend through LODs, observe terrain and local vegetation.

Locate: choose a simulation location, fly to it, observe local environmental and population state.

Observe Astronomy: watch Earth rotation, Earth revolution, Moon orbit, and visually consistent solar illumination.

Run Simulation: play/pause, change bounded time speed, observe environment/life/RL telemetry, save/load deterministic world state.

Debug: switch diagnostic layers for LOD, chunk state, normals, coverage, Sun direction, day/night and performance.

## 6. Functional Requirements

| Req ID | Description |
| --- | --- |
| FR-01 | Earth renders from Solar System scale (180 units) to near-ground inspection (~0.005 units above terrain surface). |
| FR-02 | Terrain LOD adapts continuously: distant chunks coarse (LOD 0-2), local chunks fine (LOD 7-9 at near-ground). |
| FR-03 | Frustum culling eliminates out-of-view chunks every frame with zero false negatives. |
| FR-04 | Configurable chunk render distance (2.5u / 4.5u / 7.5u / Ultra/Horizon). |
| FR-05 | Load/unload hysteresis: 70% split threshold vs 125% unload margin prevents chunk thrashing at boundaries. |
| FR-06 | Multi-tier chunk cache: GPU cache -> CPU RAM cache -> async worker regeneration; returning to a visited area never re-computes mesh data. |
| FR-07 | GPU buffer pool: evicted tile GPU buffers are recycled (VBO.write) rather than deleted and re-allocated. |
| FR-08 | Asynchronous terrain worker threads: up to 3 parallel workers; max 6 GPU uploads per frame budget. |
| FR-09 | Ancestor tile fallback: if a child tile is still building, the nearest resident ancestor renders in its place (no holes). |
| FR-10 | Dynamic near clip plane: adapts from 0.5 at orbit to 0.001 at near-ground to prevent terrain clipping. |
| FR-11 | Terrain clearance: camera min_dist updates every frame from actual displaced terrain elevation under the camera. |
| FR-12 | Earth rotation (23.44 deg axial tilt), orbital mechanics (365.25 d), and Moon orbit (27.32 d, 5.14 deg inclination) are deterministic and immutable. |
| FR-13 | Sun illumination: correct NdotL lighting, day/night terminator, atmospheric limb scattering, and night city glow. |
| FR-14 | 10 visual debug modes: Normal PBR, Earth Solid, Wireframe, Heightmap, Biome, Land Mask, Ocean Mask, Normals, LOD Color, Tile Bounds. |
| FR-15 | Simulation scheduler: multi-rate ticking (hourly, 6-hourly, daily, monthly) decoupled from render loop. |
| FR-16 | Environment engine: surface energy balance, Coriolis wind, orographic precipitation, soil moisture, vegetation biomass. |
| FR-17 | Human cohort: physiological homeostasis, foraging, knowledge discovery, cultural transmission, generational demographics. |
| FR-18 | RL interface: 12-dimensional normalized observation, 14 discrete actions with dynamic action masking, 4 interchangeable reward formulations. |
| FR-19 | World save/load: simulation instance state persists in JSON; canonical world is not saved (immutable reference). |
| FR-20 | HUD telemetry: bottom bar shows Day, Temperature, Humidity, Wind, Rain, Population, Chunks, LOD, GPU/RAM cache, FPS, Simulation Speed. |
| FR-21 | INFO card shows: 11 telemetry rows including Active Chunks, VRAM Resident, RAM Cache, Upload Queue, Fallback Tier, Render Distance. |

## 7. Non-Functional Requirements

| Attribute | Specification |
| --- | --- |
| Performance | Target 60+ FPS at 1280×768 on a dedicated GPU (NVIDIA RTX/GTX, AMD Radeon). 120 FPS cap. |
| Scalability | Supports up to LOD 9 (512× subdivision of root face) with bounded GPU/CPU memory via LRU eviction. |
| Determinism | Given the same seed, canonical world data, and simulation inputs, all outputs are reproducible. |
| Memory | GPU tile cache bounded at 384 tiles; CPU RAM cache bounded at 768 chunks (configurable). |
| Testability | 55-point architectural verification test + 6-point chunk streaming test; both must pass in headless mode. |
| Portability | Windows 10/11 primary target; OpenGL 3.3 Core required; no DirectX or Vulkan dependency. |
| Extensibility | Spatial engine, RL interface, and environment engine are cleanly separated for independent evolution. |

## 8. Out of Scope

Atom-by-atom or full GCM (Global Circulation Model) fidelity simulation.

Voxel / Minecraft block representation of terrain.

Rendering every Earth object at maximum detail simultaneously.

Replacing the canonical Earth with a random procedural planet per session.

Full satellite imagery or photorealistic texture data ingestion.

GPU acceleration as a hard dependency of the simulation/data layer.

## 9. Product Acceptance Criteria

Earth renders visually complete from global scale (180u) through descent to near-ground (<0.01u above surface).

Zoom reaches near-ground terrain without underground clipping, NaN/Inf errors, or renderer hangs.

Local detail (LOD 7–9) increases only where camera attention requires it; distant chunks remain coarse.

Sun/Earth/Moon motion is coherent and visually coupled to real-time lighting direction.

Chunk streaming: returning to a visited area uses cached geometry with zero re-generation.

Load/unload hysteresis verified: no chunk thrash at boundary over multiple camera positions.

All 55 architectural requirements pass in automated headless test.

All 6 chunk streaming requirements pass in automated headless test.

UI scroll never changes camera zoom when pointer is over left panel.

World saves and reloads reproduce simulation state deterministically.

# PART II — TECHNICAL ARCHITECTURE DOCUMENT

## 10. Architecture Overview

Immutable Canonical Earth + Astronomical Frame
        |
        +---> Spatial Engine (Cube-Sphere + Quadtree LOD Manager)
        |           +---> TerrainTilePool (Multi-Tier Chunk Cache + GPU Buffer Pool)
        |           +---> build_tile_mesh_data (CPU Mesh Builder, Async Workers)
        |           +---> EarthRenderer (Frustum Culling, Ancestor Fallback, Telemetry)
        |
        +---> Simulation Engine (WorldInstance)
        |           +---> MultiRateScheduler (Hourly/6H/Daily/Monthly)
        |           +---> EnvironmentEngine (Energy Balance, Wind, Rain)
        |           +---> HumanSettlementCohort (Vitals, Knowledge, Demographics)
        |           +---> RLLSEnvironmentInterface (Obs, Actions, Rewards)
        |
        +---> 3D Scene (SceneRenderer / ModernGL)
        |           +---> Starfield (3000 deterministic stars)
        |           +---> Sun (emissive corona shader)
        |           +---> Earth (EarthRenderer — cubesphere terrain)
        |           +---> Moon (tidal-locked orbit)
        |           +---> Orbital Lines, Spatial Grid
        |
        +---> 2D UI (pygame Surface composited over OpenGL)
                    +---> HomeScreen (Create/Load world modal)
                    +---> SimulationHUD (6 panels + bottom bar + INFO card)

## 11. Runtime Layers

| Layer | Responsibility |
| --- | --- |
| L0: OS / Hardware | Windows 10/11, NVIDIA/AMD/Intel GPU, DirectX / WGL context. |
| L1: OpenGL 3.3 Core | ModernGL Python bindings; vertex/fragment shaders; VBO/VAO/FBO. |
| L2: Graphics Engine | SceneRenderer, EarthRenderer, TerrainTilePool, LODManager, OrbitCamera. |
| L3: Simulation Engine | WorldInstance, MultiRateScheduler, EnvironmentEngine, HumanSettlementCohort. |
| L4: RL Interface | RLLSEnvironmentInterface (Gym-style), 12-dim obs, 14 actions, 4 reward modes. |
| L5: UI Layer | HomeScreen + SimulationHUD layered as pygame SRCALPHA surface over L2. |
| L6: Persistence | canonical_world.npz (immutable), simulations/<name>/world_state.json (mutable). |

## 12. Canonical World Model

The canonical world dataset (worlds/canonical_world.npz) is generated once by generate_world.py and never modified at runtime. It is a NumPy archive containing:

| Field | Description |
| --- | --- |
| elevation | float32 [H, W] — normalized 0.0..1.0, sea level at 0.50. Encoded uint16 [0..65535]. |
| land_mask | bool [H, W] — True = land, False = ocean. Strictly calibrated to 29.2% land / 70.8% ocean. |
| temperature | float32 [H, W] — normalized latitudinal+altitude temperature field 0..1 (~-35°C to +35°C). |
| precipitation | float32 [H, W] — normalized annual precipitation 0..1 (ITCZ, subtropical dry belts, midlat storm tracks). |
| biome | uint8 [H, W] — 0=Ocean, 1=Ice, 2=Desert, 3=Grassland, 4=Forest, 5=Rainforest, 6=Tundra. |
| hydrology | float32 [H, W] — drainage potential 0..1 (precipitation × inverse-slope × inverse-altitude). |
| vegetation | float32 [H, W] — biomass potential 0..1 (temperature suitability × water suitability). |
| latitude | float32 [H] — grid row latitudes in radians [-π/2, π/2]. |
| longitude | float32 [W] — grid column longitudes in radians [-π, π]. |

Resolution: 512 × 1024 cells (configurable via PlanetConfig). Canonical ID = hash(schema_version + dataset_version).

## 13. Spatial Representation — Cube-Sphere Quadtree

The terrain rendering system uses a 6-face normalized cube-sphere. Each face carries an independent quadtree managed by LODManager. Cube-face UV coordinates (u, v) map to normalized sphere coordinates via the cube_to_sphere() function, avoiding the latitude-convergence distortion of equirectangular grids.

Face 0 = +X, Face 1 = -X, Face 2 = +Y (north), Face 3 = -Y (south), Face 4 = +Z, Face 5 = -Z.

Root node: u ∈ [-1, 1], v ∈ [-1, 1]. Level 0 = 6 root tiles covering the whole planet.

Each subdivision halves both u and v extents, producing 4 child tiles (bottom-left, bottom-right, top-left, top-right).

bounding_radius per node ≈ chord half-length on unit sphere from center to corner.

center_sphere = cube_to_sphere(face, center_u, center_v) — used for horizon culling and frustum tests.

Node key = f"{face}_{level}_{u_min:.5f}_{v_min:.5f}" — globally unique, stable, cache-addressable.

## 14. Terrain Tile Pipeline

Request tile key
  -> Tier 1: GPU Cache hit (OrderedDict LRU)    -> render immediately
  -> Tier 2: CPU RAM Cache hit                  -> upload to GPU with recycled buffer -> render
  -> Tier 3: LOD 0 nodes                        -> synchronous build_tile_mesh_data -> CPU+GPU cache
  -> Tier 4: LOD ≥ 1 nodes                      -> enqueue to ThreadPoolExecutor worker
                                                 -> worker calls build_tile_mesh_data
                                                 -> places (key, verts, indices) in ready_cpu_meshes queue
             Main render thread: process_ready_uploads() drains queue (max 6 per frame)
                                                 -> upload with buffer recycling
                                                 -> store in CPU + GPU cache
  -> Tile absent while building: walk parent chain -> nearest resident ancestor renders instead

## 15. Deep-Zoom LOD Architecture

LODManager.max_lod scales dynamically with altitude: LOD 6 at base, up to LOD 9 when camera altitude < 0.04 units (~250m).

Screen-space error metric: projected_error = (geometric_error / dist_to_node) × proj_scale where proj_scale = (viewport_height / 2) / tan(fovy/2).

Split when projected_error > split_threshold (1.0 × error_threshold=3.5).

Merge when projected_error < merge_threshold (0.70 × error_threshold). Prevents thrashing at boundaries.

Geographically targeted: only nodes near the camera and inside the frustum subdivide to high LOD.

At LOD ≥ 2: multi-octave FBM refinement (seed=16001+level, 3 octaves) adds micro-scale fractal land detail.

At LOD ≥ 5: additional high-frequency micro-crag noise (seed=32003+level, 2 octaves) for near-ground terrain.

Perimeter skirts on every tile eliminate cracking at LOD boundaries between neighboring tiles.

## 16. Terrain Detail Strategy

build_tile_mesh_data() produces a 16×16 vertex grid per tile (256 vertices + perimeter skirt vertices) with vertex layout [x, y, z, nx, ny, nz, u_tex, v_tex, elevation, land_mask] (10 floats/vertex). Surface normals are computed via central differences of the displaced mesh positions for genuine 3D relief lighting on mountains, ridges, valleys, and craters — not purely radial normals.

| LOD Range | Terrain Detail Description |
| --- | --- |
| LOD 0–1 | Root tiles. Coarse canonical elevation only. Radial-blended normals (35–70% computed + 30–65% radial). |
| LOD 2–4 | Canonical elevation + 3-octave FBM land refinement (scale 4×2^(level-2)), amplitude 0.04/2^(level-1). |
| LOD 5–9 | LOD 2–4 detail + high-frequency micro-crag noise (scale 16×2^(level-4)), amplitude 0.012/2^(level-1). Full central-diff normals. |

## 17. Frustum Culling System

Each frame, EarthRenderer extracts 6 view-projection frustum planes using extract_frustum_planes(VP) from math3d.py. LODManager.update() receives these planes and culls all nodes whose bounding sphere (center in world space, radius = terrain_margin + node_radius_world) lies entirely outside any plane.

Planes extracted from combined VP = proj @ view matrix (row-major, column-major OpenGL convention).

Planes normalized so dot(normal, point) + D gives signed distance; node is culled if dist < -radius.

Horizon culling: node center_sphere dot cam_dir_model < horizon_dot_limit − bounding_radius − margin.

Both culling passes run in model space to handle Earth's axial tilt and diurnal rotation correctly.

culled_node_count incremented for each culled subtree; visible in telemetry.

## 18. Astronomy and Coordinate Frames

Simulation time (sec) -> compute_astronomical_state() -> {
    earth_pos    : vec3 (orbit radius=90u, period=365.25d)
    earth_rot    : float (diurnal rotation 24h period)
    moon_pos     : earth_pos + orbit(12u radius, 27.32d, 5.14° inclination)
    moon_rot     : tidal-locked = moon_orbit_phase
    season       : quadrant of orbit_phase
}

Coordinate convention: right-handed, Y = up, +X = east, +Z = south. Earth axial tilt: rotate_z(-23.44°) applied to model matrix. Diurnal rotation: rotate_y(earth_rot_angle) applied on top of tilt. Astronomical constants are frozen in CanonicalCelestialSystem (frozen dataclass) — immutable at runtime.

## 19. Lighting Pipeline

Sun at world origin [0,0,0]
  -> compute sun_pos relative to Earth body: u_sun_pos = vec3(0,0,0)
  -> L = normalize(sun_pos - world_pos)
  -> NdotL = dot(N, L)
  -> daylight = smoothstep(-0.08, 0.12, NdotL)   [smooth day/night terminator]
  -> lit_color = surface_color * (daylight * solar_irradiance + night_ambient=0.035)
  -> Ocean specular: pow(NdotH, 48..192) * NdotL * 0.70 * solar_irradiance
  -> Atmosphere limb: pow(1-NdotV, 3.2) * atmo_color * 0.65

Note: In v0.3 the cubesphere terrain shader correctly receives u_sun_pos = (0,0,0) as the Sun is at the world origin. The model matrix correctly positions Earth in its orbit. This is geometrically correct: the Sun direction in world space is simply -normalize(earth_world_pos) since Sun is at origin.

## 20. Moon Rendering

Moon orbit computed from simulation time: sidereal period 27.32166 days, inclination 5.14° to ecliptic.

Moon rotation = orbit phase (tidal locking — always same face toward Earth).

Lunar texture generated procedurally: basaltic mare basins + highland regolith grayscale.

Moon rendered as smooth UV sphere (36×72 segments) with prog_moon diffuse shader.

Moon radius 1.4 rendering units; hit-box for double-click focus: sphere radius 2.5.

## 21. GPU Memory Architecture

| Resource | Specification |
| --- | --- |
| GPU Tile Cache | OrderedDict LRU, max 384 tiles (dedicated GPU) or 256 tiles (integrated). ~0.022 MB/tile VRAM estimate. |
| CPU RAM Cache | OrderedDict LRU, max 768 chunks. Geometry (vertices, indices) arrays in system RAM. Zero re-generation on cache hit. |
| Free Buffer Pool | List of evicted TerrainTile objects with pre-allocated VBO/IBO. Max pool size 128. vbo.write() / vbo.orphan() reuse. |
| Upload Budget | Max 6 GPU tile uploads per frame. Ready meshes drained from thread-safe queue. Prevents stutter. |
| Worker Threads | ThreadPoolExecutor with 3 workers. LOD 0 tiles built synchronously. LOD ≥ 1 tiles built asynchronously. |
| Emergency Sphere | Pre-baked UV sphere (48×96 segments) in separate VBO always ready as fallback if tile system fails. |

## 22. Shader System

| Shader | Purpose |
| --- | --- |
| CUBESPHERE_TERRAIN_VS/FS | Main Earth terrain shader. Receives per-vertex [x,y,z,nx,ny,nz,u,v,elev,land]. 10 debug modes. Deep-zoom micro-detail. |
| PLANET_VS / EARTH_FS | Emergency sphere fallback and base Earth shader with cloud layer, atmosphere, specular. |
| MOON_FS | Lunar diffuse shader with regolith texture. |
| SUN_VS / SUN_FS | Emissive sun with limb darkening, granulation pattern, corona fringe. |
| STARFIELD_VS/FS | 3,000 point stars with brightness-modulated color; translation stripped from view matrix (depth = 1.0). |
| LINE_VS/FS | Orbital paths and spatial grid with configurable RGBA color. |
| OVERLAY_VS/FS | Fullscreen quad for pygame UI surface compositing (RGBA blending). |

## 23. Multi-Scale Camera

| Property | Detail |
| --- | --- |
| Distance range | 0.005 (near-ground) to 800.0 (deep solar system) units. Logarithmic zoom step ∝ altitude. |
| Focus targets | SOLAR (origin), SUN (at origin), EARTH, MOON. Focus snaps or lerps to target. |
| Min distance | Dynamically set from displaced terrain elevation under camera: surf_r + 0.005 (≈30–50m above terrain). |
| Near clip plane | near = max(0.001, altitude × 0.08). Smoothly prevents terrain clipping. |
| Orbit controls | Left-drag: yaw/pitch. Middle/right-drag: pan. Wheel: altitude-proportional zoom. |
| Keyboard controls | W/A/S/D: pan; Q/E: vertical; +/-: zoom; F: focus Earth; Space: pause; ESC: reset. |
| Double-click focus | Ray-cast from screen pixel; hits Sun (r=13), Earth (r=6), Moon (r=2.5); focus nearest hit. |
| Fly-to animation | fly_to_lat_lon(): aim yaw/pitch at geographic lat/lon, lerp over duration seconds. |
| Motion smoothing | dt-independent exponential decay: decay = 1 - exp(-rate × dt). Rate 8.5 (pos), 10.0 (rot), 9.0 (dist). |

# PART III — SECURITY AND DATA INTEGRITY SPECIFICATION

## 24. Canonical Data Immutability

canonical_world.npz is loaded read-only at startup and never written during simulation.

WorldInstance holds a reference to world_data dict; it must not mutate any canonical array in-place.

Simulation resets restore mutable state from initial parameters; canonical data is unchanged.

Canonical dataset updates require an explicit version change and full validation report.

Save files must record the canonical-world version identifier so stale data is detected on load.

## 25. Simulation State Persistence Security

World names are sanitized (alphanumeric + hyphens/underscores) before being used as directory paths.

Resolved save path must remain under the simulations/ root directory (no path traversal).

World state JSON is validated on load: required keys, field dtypes, and value ranges are checked.

Never execute arbitrary Python code from world files. JSON schema only.

Atomic write strategy: write to temporary file, then rename, to prevent corruption on crash.

## 26. File and I/O Limits

canonical_world.npz: file size < 200 MB enforced before loading. Field shapes validated against expected [H, W].

World state JSON: max 1 MB; reject oversized files.

Do not load externally sourced datasets without explicit decompression and size limits.

## 27. GPU Resource Safety

GPU buffers released on application exit via tile_pool.release_all() and emergency_vbo/ibo.release().

TerrainTile.release() wrapped in try/except to prevent crash on driver context loss.

ThreadPoolExecutor shut down with wait=False on exit (non-blocking); background tasks may be interrupted.

No ModernGL objects are created or used from worker threads (GPU API calls are main-thread only).

Buffer orphan strategy (vbo.orphan) used when buffer size changes rather than creating new allocation.

## 28. Secrets and Credentials

No API keys, tokens, or cloud credentials are stored in source, world saves, or canonical data.

Use environment variables or platform-appropriate secret stores for optional integrations.

Diagnostics and logs must never expose secrets, passwords, or private data.

## 29. Dependency Security

Requirements pinned or constrained: moderngl>=5.8.2, pygame>=2.5.2, numpy>=1.26.4, scipy>=1.13.1.

Review transitive dependencies when adding libraries.

Do not bundle virtual environments into source repositories.

Remove obsolete dependencies after replacement.

## 30. Security Acceptance Criteria

Path traversal tests pass (world names with '../' rejected).

Malformed canonical world files fail safely with clear error messages.

Canonical world cannot be modified through ordinary runtime APIs.

No secrets appear in repository files, diagnostics, or telemetry.

All major code paths testable in headless mode without external network access.

# PART IV — FRONTEND SPECIFICATION DOCUMENT

## 31. Frontend Architecture

The UI is a workstation-style interface layered as a fully transparent pygame SRCALPHA surface composited over the ModernGL 3D viewport via a fullscreen textured quad (OVERLAY_VS/FS). Input routing is deterministic: events are hit-tested against UI zones first; only events that miss all UI zones are forwarded to the 3D camera. This guarantees panel scroll never changes camera zoom.

## 32. Screen Map

Home Screen
  -> Create World Modal (ease-out slide in)
  -> [Loading Checklist: 7 stages with live progress]
  -> Main Simulation Workstation
       |-- Top Bar            (simulation time, play/pause, 8 speed multipliers)
       |-- Left Control Panel (7 collapsible sections, 200ms ease-out animation)
       |-- Central 3D Viewport (ModernGL — fills remaining width×height)
       |-- Right Toolbar      (FOCUS / EARTH / ORBIT / GRID / INFO / MEASURE / LOCATE)
       |-- Bottom Status Bar  (telemetry bar: Day, Temp, Humidity, Wind, Rain, Population,
       |                       Chunks [LOD], GPU/RAM cache, FPS, Sim Speed)
       |-- INFO Card          (toggleable overlay: 11 telemetry rows)

## 33. Home Screen

Title card: RLLS 16 logo, version label, brief description.

CREATE WORLD button → opens Create World Modal with ease-out animation.

LOAD WORLD button → file-picker for existing simulations/<name>/world_state.json.

EXIT button.

No 3D rendering occurs on Home Screen; background is solid dark.

## 34. Create World Modal

Fields: World Name (text), Initial Population (slider 100–100,000), Environment Archetype (10 presets), RL Goal.

10 environment archetypes: Forest, Grassland, Savannah, Desert, Tundra, Mountain, Coastal, River Valley, Tropical, Temperate.

On CONFIRM: triggers 7-stage loading checklist before entering simulation.

Loading stages: (1) Load canonical Earth, (2) Validate fields, (3) Initialize environment engine, (4) Deploy human settlement, (5) Start scheduler, (6) Warm GPU cache, (7) Enter simulation.

Modal and checklist animate in with ease-out cubic easing (200ms).

## 35. Left Control Panel

| Section | Controls |
| --- | --- |
| WORLD | Save / Reset / Return to Menu buttons. Shows world name. |
| WORLD / SPACE | Read-only astronomical constants: Sun luminosity, solar constant, Earth orbit, axial tilt, Moon orbit. |
| EARTH | 4 interactive sliders: Solar Irradiance (0.5–1.5×), Temp Offset (±10°C), Cloud Cover (0–2×), CO2 ppm (280–880). |
| LIFE | Population Dynamics panel (pop, birth/death rate, food security, shelter). Individual Physiology panel (health, hydration, energy, thermal comfort). |
| REINFORCEMENT LEARNING | Active Goal toggle (Survive / Growth / Exploration / Culture). Observation/action space summary. Action mask display. |
| VISUALIZATION | Debug view cycle (10 modes). Chunk render distance toggle (2.5u / 4.5u / 7.5u / Ultra). Atmosphere / Clouds / Orbits / Grid toggles. |
| DATA & ANALYTICS | 6 sparkline microcharts: Temperature, Population, Rainfall, Biomass, Water/Soil, Health (60-sample rolling history). |

## 36. Right Navigation Toolbar

| Button | Action |
| --- | --- |
| FOCUS | Cycle focus: Solar System → Earth → Moon → Sun. |
| EARTH | Instantly focus Earth at distance 24. |
| ORBIT | Toggle orbital path lines on/off. |
| GRID | Toggle spatial coordinate grid on/off. |
| INFO | Toggle 11-row SYSTEM & TELEMETRY info card. |
| MEASURE | Toggle distance measure mode. |
| LOCATE | Fly camera to current human settlement coordinates. |

## 37. Bottom Status Bar Specification

The bottom status bar (height 28 px) renders in a single text line at all times:
DAY {day} | {temp}°C | Humidity {h}% | Wind {w} m/s | Rain {r} mm/h | Population {pop} | CHUNKS {n} [LOD{max_lod}] GPU:{gpu_cache} RAM:{ram_cache} | FPS {fps} | SIM {speed}

## 38. INFO Card Telemetry (11 Rows)

| Row | Content |
| --- | --- |
| Central Body | Sun (Type G2V) |
| Earth Distance | 1.000 AU (149.6M km) |
| Moon Phase | Orbital angle in degrees from simulation time |
| Active GPU | GPU renderer name (from GL_RENDERER) |
| Render Rate | FPS and frame time in ms |
| Chunk Dist | Current render distance setting (2.5u / 4.5u / 7.5u / Ultra) |
| Active Chunks | Rendered tiles / culled tiles [max LOD] |
| VRAM Resident | GPU cache tile count and estimated VRAM MB |
| Chunk Cache | RAM cache size / Pool size / reused buffer count |
| Upload Queue | Pending background mesh builds |
| Fall-back Tier | HIGH_DETAIL_CUBESPHERE / LOW_DETAIL / EMERGENCY_SPHERE |

## 39. Input Routing Specification

Event arrives
  -> hit-test: my < 48           -> Top Bar consumed
  -> hit-test: my > height - 28  -> Bottom Bar (no interaction, read-only)
  -> hit-test: mx < panel_x + panel_width AND panel open  -> Left Panel consumed
  -> hit-test: mx > width - 70   -> Right Toolbar consumed
  -> hit-test: INFO card visible AND in card rect         -> INFO card consumed
  -> otherwise: route to 3D camera

Mouse wheel events: if pointer inside left panel → scroll panel (bounded). Else → camera zoom. Never both.

## 40. Visual/Interaction Acceptance

No accidental camera zoom from panel scrolling.

Panel scroll is bounded (scroll_y clamped); no infinite scroll.

Left panel slides in/out with 200ms ease-out cubic animation (never teleports).

Notification banners appear centered at y=60 with card background.

All interactive buttons have minimum 44×44 px hit areas.

HUD remains readable at all zoom levels (no text overflow or Z-fighting).

Debug modes are cycle-accessible via Prev/Next buttons in VISUALIZATION section.

# PART V — CHUNK STREAMING & DEEP ZOOM ARCHITECTURE

## 41. Overview

The Minecraft-inspired chunk streaming architecture (implemented in v0.3) transforms the Earth renderer from a single-pass brute-force LOD system into a spatially-aware, memory-bounded, asynchronous tile streaming pipeline. All terrain data is preserved across render-distance boundaries; chunks are only unloaded from active GPU draw calls, never from the CPU RAM cache.

## 42. LOD Manager — Quadtree Update Algorithm

LODManager.update() per-frame algorithm (called once per EarthRenderer.render() call):

For each of 6 root faces:
  traverse(node):
    1. Horizon cull: dot(node.center_sphere, cam_dir_model) < limit - r - margin  -> skip subtree
    2. World-space transform: rotate node center from model space to world space
    3. Frustum cull: for each of 6 planes: dist(plane, node_world) < -node_radius -> skip subtree
    4. Render distance check with hysteresis:
         limit = render_dist × 1.25 if node was active last frame else render_dist
         if dist_surface > limit: skip subtree
    5. Screen-space error:
         chord = radius × 2 / 2^level
         sagitta = chord² / (8 × radius)
         geometric_error = sagitta + 0.35 / 2^level
         projected_error = (geometric_error / dist) × proj_scale
    6. Split/Merge with hysteresis:
         if leaf and projected_error > split_threshold and level < max_lod: subdivide + recurse
         if !leaf and projected_error < merge_threshold: collapse to leaf
         else: recurse children

## 43. GPU Buffer Pool Recycling Protocol

When a GPU tile is evicted from the LRU GPU cache:

Tile is placed in free_buffer_pool (list of TerrainTile objects) if pool size < 128.

When a new tile needs GPU upload, pop from free_buffer_pool if available.

Call tile.update_data(verts, indices, key) — writes verts/indices into existing VBO/IBO via vbo.write().

If new data is larger than current buffer: vbo.orphan(new_size) first (invalidates buffer, no copy).

Reused tile is placed in GPU cache under new key. reused_buffers_count incremented.

If free_buffer_pool is empty: instantiate new TerrainTile (new ctx.buffer() calls).

This eliminates OpenGL driver allocation stalls for steady-state rendering.

## 44. Hysteresis Parameters

| Parameter | Value / Rule |
| --- | --- |
| split_threshold | 1.0 × error_threshold (3.5). Node subdivides when projected SSE exceeds this. |
| merge_threshold | 0.70 × error_threshold (2.45). Non-leaf collapses only when SSE drops below this. |
| load_render_dist | User-configured (2.5 / 4.5 / 7.5 units) or automatic horizon distance. |
| unload_render_dist | 1.25 × load_render_dist. Chunks that were active last frame stay active until this margin. |
| max_uploads/frame | 6 tile GPU uploads maximum per render frame. Prevents stutter spikes. |
| worker_threads | 3 parallel terrain mesh builder workers (ThreadPoolExecutor). |
| max_lod_schedule | altitude < 0.04u → max_lod=9; < 0.12u → 8; < 0.35u → 7; < 1.2u → 6; else → 5. |

## 45. Camera Deep Zoom Specification

Zoom range: 800.0 units (outer solar system) to surf_r + 0.005 (≈30–50m above displaced terrain).

zoom step = max(0.003, altitude × 0.15). Logarithmically shrinks as camera approaches surface.

Ground clearance: every frame, sample_terrain_altitude(lat, lon, world_data) returns displaced surface radius at camera's current lat/lon. min_dist = surf_r + 0.005.

Near clip plane: near = max(0.001, altitude × 0.08). Clamps to 0.001 minimum to prevent Z-fighting.

Far clip plane: 3000.0 units (constant — sufficient for solar system scale).

Orbit sensitivity scales logarithmically with altitude: low-orbit panning is slow/precise; orbit panning is wide.

Pan speed = 0.0007 × altitude. Keyboard pan speed = 18 × dt × altitude/20.

## 46. Shader Micro-Detail at Deep Zoom

When camera is within 3.85 units of the surface (detail_factor > 0, computed per-fragment as clamp(1 - (dist_to_cam - 0.05) / 3.8, 0, 1)), the fragment shader applies additional micro-detail:

| Effect | Implementation |
| --- | --- |
| Slope detection | slope = 1 - dot(N, normalize(world_pos)). High slope = steep cliff face. |
| Cliff striations (land) | rock_strata = sin(elev × 800 + sin(u × 2000) × 0.5) × 0.5 + 0.5; blended with smoothstep(0.08, 0.28, slope) × detail_factor. |
| Micro-grain (land) | micro_uv = uv × 2048; grain = sin(micro_uv.x × π + sin(micro_uv.y × 4.2)) × cos(...)×0.10; surface_color *= (1 + grain × detail_factor). |
| Ocean ripples | wave = sin(ocean_uv.x + cloud_offset × 120) × cos(ocean_uv.y + cloud_offset × 90); surface_color += blue_tint × wave × detail_factor. |
| Specular sharpening | spec_exp = mix(48, 192, detail_factor) for distance-adaptive ocean specular glint. |

## 47. Verification Tests

| Test File | Coverage |
| --- | --- |
| test_55_points_verification.py | 7 verification groups covering all 55 architectural requirements. Runs headless with OpenGL. Must pass 100%. |
| test_chunk_streaming_verification.py | 6 streaming tests: 1. Frustum plane extraction and sphere-in-frustum math. 2. LODManager frustum culling (culled_count > 0 at 5.5u distance). 3. Deep zoom: camera reaches dist < 6.0u, near < 0.05, clearance correct. 4. Distance-based LOD: orbit → max_lod ≤ 3; ground → max_lod ≥ 6. 5. Configurable render distance: short (1.0u) < wide (5.0u) chunk count. 6. Load/unload hysteresis: boundary chunks preserved across step. 7. Multi-tier cache + GPU buffer recycling (reused_buffers ≥ 1). 8. Full EarthRenderer integration: rendered_tiles > 0 at all scales. |

# PART VI — FEATURE TICKET LIST AND ENGINEERING RULES

## 48. Implementation Status (v0.3)

| Module | Status |
| --- | --- |
| math3d.py | COMPLETE — extract_frustum_planes, sphere_in_frustum, sample_terrain_altitude, all standard matrix ops. |
| quadtree.py | COMPLETE — LODManager with frustum culling, render distance, hysteresis (70%/125%), dynamic LOD 0–9. |
| terrain_tile.py | COMPLETE — Multi-tier cache, GPU buffer pool, async workers, surface normals, FBM refinement, perimeter skirts. |
| camera.py | COMPLETE — Deep zoom to 0.005u above terrain, terrain clearance sensing, dynamic near clip, logarithmic zoom. |
| shaders.py | COMPLETE — 10 debug modes, slope-aware cliff striations, micro-grain, ocean ripples, atmosphere limb. |
| renderer.py | COMPLETE — Frustum extraction per frame, ancestor fallback, full telemetry dict, emergency sphere tier. |
| hud.py | COMPLETE — Render distance toggle, expanded INFO card (11 rows), bottom bar chunk telemetry. |
| diagnostics.py | COMPLETE — 10-stage graphics pipeline diagnostic. |
| gpu_config.py | COMPLETE — Windows GPU preference, MSAA, anisotropic filtering, hardware detection. |
| astronomy.py | COMPLETE — Deterministic orbital mechanics, CanonicalCelestialSystem frozen dataclass. |
| environment_engine.py | COMPLETE — Energy balance, Coriolis wind, orographic precipitation, soil/vegetation dynamics. |
| human_population.py | COMPLETE — Physiological homeostasis, foraging, cultural lore, generational demographics. |
| rl_environment.py | COMPLETE — 12-dim obs, 14 actions, dynamic action masking, 4 reward formulations. |
| tests/ | COMPLETE — 55-point + 6-point chunk streaming verification suites. Both pass 100%. |

## 49. P0 — Backlog / Known Limitations

| Ticket | Description |
| --- | --- |
| RLLS-P0-01 | Vegetation / surface objects: trees, rocks, buildings are not yet rendered as instanced geometry. Currently only biomass field is simulated. |
| RLLS-P0-02 | Local high-resolution texture tiles: deep-zoom terrain uses procedural FBM; no real satellite imagery or photogrammetry tiles. |
| RLLS-P0-03 | Camera prefetch forward cone: camera approaching a location does not yet prefetch the tile chain ahead of arrival. |
| RLLS-P0-04 | Sun position in Earth shader: sun_pos is always (0,0,0) in world space (correct as Sun is at origin), but earth_pos in orbit means the sun direction per-fragment should be normalized(earth_pos - world_pos). Currently uses normalize(-world_pos) which is equivalent but worth documenting. |
| RLLS-P0-05 | RL policy training: no bundled agent trainer. Interface is Gym-compatible but training loop is external. |

## 50. P1 — Near-Term Improvements

| Ticket | Description |
| --- | --- |
| RLLS-P1-01 | Vegetation instancing: GPU-instanced tree/shrub/grass billboards at LOD ≥ 6 tiles when biomass > threshold. |
| RLLS-P1-02 | Normal maps: add per-tile normal map textures at LOD ≥ 4 for higher-frequency surface detail without mesh geometry. |
| RLLS-P1-03 | Water bodies: render lakes/rivers as reflective planes at deep-zoom levels using canonical hydrology field. |
| RLLS-P1-04 | Camera forward-cone prefetch: when fly_to_lat_lon() is called, immediately submit tile requests along the flight corridor. |
| RLLS-P1-05 | Sun light direction fix: pass actual sun-to-fragment direction to improve shadow terminator accuracy at non-origin Earth positions. |
| RLLS-P1-06 | Cloud shadow: project cloud texture shadow onto terrain surface using sun direction. |

## 51. P2 — Future Enhancements

| Ticket | Description |
| --- | --- |
| RLLS-P2-01 | Oceans: GPU fluid simulation or displacement-mapped water surface with FFT waves. |
| RLLS-P2-02 | Multi-agent RL: support multiple human cohorts with inter-settlement trade and conflict. |
| RLLS-P2-03 | Time-lapse playback: record simulation states at intervals and replay at accelerated speed with morphing terrain. |
| RLLS-P2-04 | Export: GLB/OBJ export of visible tile meshes for use in external 3D tools. |
| RLLS-P2-05 | Atmospheric scattering: physically-based Rayleigh + Mie scattering for realistic sky color at horizon. |

## 52. Engineering Rules for Future Changes

Always read the full relevant code path before editing any file.

Prefer an existing correct subsystem over parallel reimplementation.

Use libraries for generic infrastructure when they are actually superior and maintained.

Keep canonical Earth authoritative and immutable at runtime; never write to it.

Never turn the global Earth into a maximum-detail mesh just to solve deep zoom.

Never hide renderer failures behind an emergency fallback without telemetry.

Never claim 120 FPS without measured runtime telemetry from a real device.

Never add a feature without at least one automated test for its core behavior.

Every optimization must have a measurable acceptance criterion before and after.

All behavior affecting determinism must use explicit seed/RNG ownership.

GPU API calls must only occur on the main render thread.

Do not modify TerrainTilePool.cpu_cache on eviction — only GPU cache may evict.

Worker thread exceptions must be caught, logged, and never silently swallowed.

LODManager.update() must be called once per frame with current frustum planes.

New shaders must preserve all existing u_debug_mode behaviors (0–9).

## 53. Reference Implementations / Research Inputs

The architecture intentionally draws on established planetary rendering patterns. These references are algorithmic inspirations, not verbatim sources.

Cube-sphere terrain subdivision: Outerra Engine (J. Benes et al.), Cesium.js planet renderer.

Screen-space error metric: ROAM (Real-time Optimally Adapting Meshes — Duchaineau 1997), vterrain.org.

Chunk streaming & GPU buffer recycling: Minecraft Java Edition region file format; Voxel Farm terrain engine.

Frustum plane extraction: Gribb & Hartmann 2001 — 'Fast Extraction of Viewing Frustum Planes from the World-View-Projection Matrix'.

LOD hysteresis: GPU Pro 5 — 'Terrain Rendering in Far Cry 5'; Naughty Dog GDC talks on planet-scale terrain.

Surface energy balance model: Hartmann (2016) 'A Climate for Life'; IPCC AR6 climate model parameterizations.

Human population dynamics: Malthusian/logistic growth; Axelrod cultural evolution model.

RL interface design: OpenAI Gym API (Brockman et al. 2016); multi-objective RL (Vamplew et al.).

## 54. Final Definition of Done for RLLS16 Core

RLLS16 core is complete when:

The canonical Earth is continuously inspectable from planetary scale to near-ground scale (altitude < 0.02u).

Earth, Sun, Moon, and stars render in a physically-plausible configuration with correct illumination direction.

Chunk streaming verifiably works: cache hits, buffer recycling, hysteresis, frustum culling confirmed by automated tests.

The simulation runs indefinitely without memory leaks, GPU VRAM growth, or CPU spike accumulation.

World save/load is deterministic and schema-validated.

RL interface is gym-compatible with observable, reproducible episode rollouts.

The 55-point architectural test and the 6-point chunk streaming test both pass 100% in headless mode.

All UI requirements (hitboxes, scroll isolation, animation, telemetry) are verified by the 55-point test.

# APPENDIX A — Module and File Reference

| File / Module | Responsibility |
| --- | --- |
| main.py | Entry point. Calls RLLS16App(canonical_path).run(). |
| generate_world.py | Runs planet generation; saves canonical_world.npz. |
| view_world.py | Quick visualization of canonical fields (matplotlib). |
| rlls16/app.py | RLLS16App: top-level coordinator. State machine: HOME → SIMULATION. |
| rlls16/planet.py | generate_planet(): fBm continents, ridged mountains, biome classification. |
| rlls16/noise.py | fbm_2d(), ridged_fbm(): deterministic NumPy fractal noise. |
| rlls16/storage.py | load_world() / save_world(): canonical_world.npz I/O. |
| rlls16/config.py | PlanetConfig dataclass: lat/lon resolution, seeds, terrain amplitude. |
| rlls16/graphics/math3d.py | All 4×4 matrix math + frustum extraction + terrain altitude sampling. |
| rlls16/graphics/camera.py | OrbitCamera: multi-scale logarithmic camera with terrain clearance. |
| rlls16/graphics/cubesphere.py | cube_to_sphere(), sphere_to_latlon(), sample_canonical_field(). |
| rlls16/graphics/quadtree.py | QuadtreeNode, LODManager: SSE-based LOD with frustum + render distance. |
| rlls16/graphics/terrain_tile.py | build_tile_mesh_data(), TerrainTile, TerrainTilePool. |
| rlls16/graphics/renderer.py | EarthRenderer, SceneRenderer: full 3D scene composition. |
| rlls16/graphics/shaders.py | All GLSL vertex/fragment shader source strings. |
| rlls16/graphics/gpu_config.py | GPU environment hints, MSAA config, hardware detection. |
| rlls16/graphics/diagnostics.py | 10-stage graphics pipeline diagnostic suite. |
| rlls16/simulation/astronomy.py | compute_astronomical_state(), CanonicalCelestialSystem. |
| rlls16/simulation/scheduler.py | MultiRateScheduler: hourly/6H/daily/monthly tick dispatch. |
| rlls16/simulation/environment_engine.py | EnvironmentEngine: energy balance, wind, precipitation. |
| rlls16/simulation/human_population.py | HumanSettlementCohort: vitals, knowledge, demographics. |
| rlls16/simulation/rl_environment.py | RLLSEnvironmentInterface: 12-dim obs, 14 actions, rewards. |
| rlls16/simulation/world_instance.py | WorldInstance: coordinates all simulation subsystems. |
| rlls16/ui/hud.py | SimulationHUD: 7 panels, right toolbar, bottom bar, INFO card. |
| rlls16/ui/home.py | HomeScreen: create/load modal with loading checklist. |
| rlls16/ui/theme.py | Color constants, font loading, draw helpers (panel, button, sparkline). |
| tests/test_55_points_verification.py | 55-point architectural verification (headless OpenGL). |
| tests/test_chunk_streaming_verification.py | Chunk streaming & deep zoom 6-test suite. |
| worlds/canonical_world.npz | Immutable canonical Earth dataset (generated once, 29.2% land). |

# APPENDIX B — Key Constants and Configuration

| Constant | Value |
| --- | --- |
| Earth radius (render units) | 5.0 |
| Earth orbit radius | 90.0 units from Sun |
| Earth axial tilt | 23.44° |
| Earth orbit period | 365.25 days |
| Earth rotation period | 24.0 hours |
| Moon orbit radius | 12.0 units from Earth |
| Moon orbit period | 27.32166 days |
| Moon orbit inclination | 5.14° |
| Solar constant | 1361 W/m² |
| Terrain displacement amplitude | 0.35 units (≈2240 km scale) |
| Sea level elevation value | 0.50 (normalized) |
| Max terrain displacement (mount) | 0.16 units above radius 5.0 → radius ~5.16 |
| LOD error threshold | 3.5 (split at 1.0×, merge at 0.70×) |
| Max GPU tile cache (dedicated) | 384 tiles |
| Max CPU RAM chunk cache | 768 chunks |
| GPU buffer pool max size | 128 tiles |
| Worker threads | 3 (ThreadPoolExecutor) |
| Max GPU uploads per frame | 6 |
| Tile grid size | 16×16 vertices per tile + perimeter skirt |
| Vertex layout | [x,y,z,nx,ny,nz,u,v,elev,land] = 10 floats/vertex |
| Canonical world resolution | 512×1024 (H×W) cells |
| Target FPS cap | 120 FPS (pygame clock.tick) |
| Star count | 3,000 (deterministic, seed=16001) |
| MSAA samples | 4× hardware multisampling |
| Anisotropic filtering (albedo) | 16× (if GPU supports) |